DC System & Electrical Installation Checklist: String Layout as per Approved Design

1. Introduction

Ensuring proper string layout in solar power plants is crucial for optimizing performance, minimizing losses, and ensuring safety compliance. This checklist provides a structured approach to verify that the string layout follows the approved design.

2. String Layout Verification Checklist

| Checklist Item | Description | Status (✔/✘) |
| --- | --- | --- |
| Compliance with Approved Design | Verify that strings are arranged exactly as per the approved layout |  |
| Correct Number of Modules per String | Ensure that the number of modules per string matches the approved design |  |
| Proper Series & Parallel Configuration | Confirm the correct arrangement of modules in series and parallel |  |
| Voltage & Current Matching | Verify that string voltage and current values align with inverter specifications |  |
| String Length Optimization | Ensure cable routing minimizes voltage drop and transmission losses |  |
| Equal String Distribution | Verify that strings are evenly distributed across multiple MPPTs |  |
| String Labeling & Identification | Ensure proper labeling of each string according to the single-line diagram (SLD) |  |
| Cable Management & Routing | Verify that cables are routed safely through conduits, cable trays, or trenches |  |
| MC4 Connector Compatibility & Crimping | Ensure that MC4 connectors are properly matched and securely crimped |  |
| Shading Avoidance | Confirm no unexpected shading due to nearby obstacles or panel misalignment |  |
| Installation of String Protection Devices | Verify the presence of string fuses, surge protection, and other safety devices |  |
| Compliance with Local Electrical Codes | Ensure adherence to IS, IEC, and NEC electrical safety standards |  |

3. Site-Specific Considerations

| Checklist Item | Description | Status (✔/✘) |
| --- | --- | --- |
| Ground Mount String Layout | Verify correct spacing and positioning of ground-mounted strings |  |
| Rooftop System Layout Compliance | Ensure alignment with available rooftop space and existing structures |  |
| Climate & Environmental Factors | Check for impacts of temperature variations on voltage and performance |  |
| Wind Load & Structural Safety | Verify cable routing minimizes exposure to wind and mechanical stress |  |

4. Compliance & Documentation

| Checklist Item | Description | Status (✔/✘) |
| --- | --- | --- |
| As-Built String Layout Report | Document actual string layout and note any deviations from the design |  |
| Electrical Testing & Verification Report | Record string voltage, current, insulation resistance, and continuity tests |  |
| Engineering Approval | Obtain approval from the design and quality assurance teams |  |
| Compliance with Electrical Standards | Ensure adherence to IS/IEC/NEC standards for system safety and performance |  |

5. Inverter & AC System Installation Checklist

| Checklist Item | Description | Status (✔/✘) |
| --- | --- | --- |
| Mounting of Inverter with Adequate Ventilation | Ensure proper mounting and sufficient airflow to prevent overheating |  |
| AC Output Wiring & Connection to Distribution Board | Verify proper wiring and secure connections to avoid electrical faults |  |
| Surge Protection & Circuit Breaker Installation | Confirm installation of necessary protection devices for safe operation |  |
| Verification of Grounding & Earthing Continuity | Ensure grounding is properly connected and tested for continuity |  |
| Labeling of All Electrical Components | Verify that all components are clearly labeled for easy identification |  |

6. Battery Storage & Hybrid System Checklist

| Checklist Item | Description | Status (✔/✘) |
| --- | --- | --- |
| Battery Terminal Connections Properly Insulated | Ensure terminals are covered and protected against short circuits |  |
| Proper Charge Controller Configuration | Verify charge controllers are set according to battery type and capacity |  |
| Ventilation & Temperature Control Measures for Battery Room | Ensure adequate airflow and temperature control for battery longevity |  |
| Safety Precautions for Lithium-Ion/Lead-Acid Batteries | Check for fire suppression, spill containment, and emergency shut-off procedures |  |

7. Conclusion

This checklist ensures that the string layout, inverter, AC system, and battery storage are installed and verified according to the approved design for safe and efficient solar plant operation. Proper documentation and inspections should be completed before commissioning.