Capacity Utilization Factor (CUF) Calculation for Rooftop and Ground Mount Solar Power Plant

# 1. Introduction

Capacity Utilization Factor (CUF) is a key performance indicator used to measure the efficiency of a solar power plant. It represents the ratio of actual energy generated by the plant to the maximum possible energy it could have generated over a given period.

# 2. Formula for CUF Calculation

CUF (%) = Total Energy Generated (kWh)Installed Capacity (kW)×Total Hours in Period×100

Where:

- Total Energy Generated (kWh) = Sum of energy produced over a period

- Installed Capacity (kW) = Total rated capacity of the PV system

- Total Hours in Period = Number of hours in a day, month, or year depending on calculation scope (e.g., 24 hours × 365 days = 8,760 hours in a year)

# 3. Example Calculation

### Given Data:

- Installed Capacity: 100 kW

- Total Energy Generated in a Year: 1,50,000 kWh

- Total Hours in a Year: 8,760 hours

CUF Calculation: CUF (%) = 150000100×8760×100 CUF (%) = 150000876000×100 CUF (%) = 17.12%

# 4. Factors Affecting CUF

- Solar Irradiation Levels: More sunlight increases energy generation.

- Panel Orientation and Tilt: Proper alignment optimizes performance.

- Shading Effects: Shading reduces output and lowers CUF.

- Temperature Coefficients: High temperatures can lower panel efficiency.

- System Losses: Wiring, inverter, and module degradation losses impact CUF.

# 5. CUF Benchmarking

| Solar Plant Type | Typical CUF Range (%) |
| --- | --- |
| Rooftop Solar | 14-20% |
| Ground Mount Solar | 17-25% |
| Solar Tracking System | 22-30% |

# 6. Conclusion

The Capacity Utilization Factor (CUF) provides a clear indication of a solar power plant’s performance. Higher CUF values indicate better efficiency and optimal system design. Regular maintenance, proper design considerations, and efficient component selection can help maximize CUF.