Battery Bank Configuration Calculation (Series & Parallel Combination) for Small- and Large-Scale Solar Power Plants

# 1. Introduction

Battery bank configuration is crucial in solar power plants to achieve the required voltage, capacity, and backup time. Proper series and parallel connections ensure optimal performance and longevity.

# 2. Key Parameters

| Parameter | Symbol | Unit | Description |
| --- | --- | --- | --- |
| Battery Voltage | Vb | V | Rated voltage of a single battery |
| Battery Capacity | Cb | Ah | Rated ampere-hour capacity of a single battery |
| Required System Voltage | Vs | V | Target voltage for the battery bank |
| Required System Capacity | Cs | Ah | Total ampere-hour requirement |

# 3. Series Connection Calculation

In a series connection, voltage increases while capacity remains the same.

### Formula:

Ns=VsVb Where:

- Ns = Number of batteries in series

- Vs = Required system voltage

- Vb = Battery voltage

### Example:

- Required system voltage: 48V

- Battery voltage: 12V Ns=4812=4 So, 4 batteries must be connected in series.

# 4. Parallel Connection Calculation

In a parallel connection, capacity increases while voltage remains the same.

### Formula:

Np=CsCb Where:

- Np = Number of parallel battery strings

- Cs = Required system capacity

- Cb = Battery capacity

### Example:

- Required capacity: 200Ah

- Battery capacity: 100Ah Np=200100=2 So, 2 parallel strings are required.

# 5. Total Battery Requirement Calculation

Ntotal=Ns×Np Where:

- Ntotal = Total number of batteries required

### Example:

- Series Batteries: 4

- Parallel Strings: 2 Ntotal=4×2=8 So, 8 batteries are required.

# 6. Configuration Table

| Parameter | Value |
| --- | --- |
| System Voltage | 48V |
| System Capacity | 200Ah |
| Battery Voltage | 12V |
| Battery Capacity | 100Ah |
| Number of Series Batteries | 4 |
| Number of Parallel Strings | 2 |
| Total Batteries Required | 8 |

# 7. IEC Standards

| Standard | Description |
| --- | --- |
| IEC 61427 | Performance testing for solar batteries |
| IEC 60896 | Stationary lead-acid battery safety standards |
| IEC 62133 | Lithium-ion battery safety requirements |
| IEC 62509 | Battery management system guidelines |

# 8. Conclusion

Battery bank configuration is an essential part of solar power system design. Proper calculations ensure system stability, longevity, and efficiency.