# Battery Capacity Sizing Calculation (Based on Load Demand & Backup Time)

## 1. Introduction

Battery capacity sizing is a critical aspect of designing energy storage systems for small- and large-scale solar power plants. Proper sizing ensures that the battery system meets the energy demands and provides adequate backup during periods of low or no solar generation. This document outlines the detailed calculation methodology, including electrical formulas and IEC standards.

## 2. Input Parameters

| Parameter | Description | Unit |
| --- | --- | --- |
| Load Demand (P) | Total power consumption of the system | kW |
| Backup Time (T) | Duration for which backup power is required | Hours |
| Battery Voltage (V) | Nominal voltage of the battery bank | Volts (V) |
| Depth of Discharge (DoD) | Maximum allowable discharge level | % |
| Battery Efficiency (η) | Efficiency of the battery system | % |
| Inverter Efficiency (η_inv) | Efficiency of the inverter | % |

## 3. Calculation Formulas

### 3.1 Energy Requirement Calculation

Ereq=P×T

where:

- Ereq = Energy required (kWh)

- P = Load demand (kW)

- T = Backup time (hours)

### 3.2 Battery Capacity Calculation

Cbatt=Ereq×100V×DoD×η×ηinv

where:

- Cbatt = Required battery capacity (Ah)

- Ereq = Energy required (kWh)

- V = Battery voltage (V)

- DoD = Depth of discharge (%)

- η = Battery efficiency (%)

- ηinv = Inverter efficiency (%)

## 4. Example Calculation (Small-Scale Solar Plant)

Given:

- Load Demand P=5kW

- Backup Time T=4hours

- Battery Voltage V=48V

- Depth of Discharge DoD=80%

- Battery Efficiency η=90%

- Inverter Efficiency ηinv=95%

Step 1: Energy Requirement Calculation

Ereq=5×4=20kWh

Step 2: Battery Capacity Calculation

Cbatt=20×10048×80×0.9×0.95

Cbatt=200032.832≈61Ah

## 5. Example Calculation (Large-Scale Solar Farm)

Given:

- Load Demand P=500kW

- Backup Time T=6hours

- Battery Voltage V=400V

- Depth of Discharge DoD=70%

- Battery Efficiency η=92%

- Inverter Efficiency ηinv=96%

Step 1: Energy Requirement Calculation

Ereq=500×6=3000kWh

Step 2: Battery Capacity Calculation

Cbatt=3000×100400×70×0.92×0.96

Cbatt=300000246.528≈1217Ah

## 6. IEC Standards & Compliance

- IEC 61427: Secondary cells and batteries for renewable energy storage

- IEC 60896: Stationary lead-acid batteries

- IEC 62619: Safety requirements for lithium-ion batteries

- IEC 62933: Electrical energy storage systems

- IEEE 485: Recommended practice for sizing lead-acid batteries for stationary applications

## 7. Conclusion

Proper battery capacity sizing ensures reliability, efficiency, and cost-effectiveness in solar power plants. The methodology outlined above provides a comprehensive approach for both small- and large-scale solar installations while adhering to industry standards.