Depth of Discharge (DoD) & State of Charge (SoC) Calculation for Battery Systems

# 1. Introduction

Depth of Discharge (DoD) and State of Charge (SoC) are crucial parameters in battery systems used in solar power plants. Proper calculation of these values ensures battery longevity, optimal performance, and efficient energy storage management.

# 2. Definitions

- Depth of Discharge (DoD): The percentage of battery capacity that has been discharged relative to its total capacity. DoD%=Energy Discharged (Wh)Battery Rated Capacity (Wh)×100

- State of Charge (SoC): The remaining battery capacity expressed as a percentage of the total capacity. SoC%=100-DoD%

# 3. Calculation Parameters

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Battery Rated Capacity | C | Wh or Ah |
| Energy Discharged | E | Wh |
| Battery Voltage | V | V |
| Battery Current | I | A |
| Depth of Discharge | DoD | % |
| State of Charge | SoC | % |

# 4. Example Calculation

### Given:

- Battery Rated Capacity = 200 Ah

- Battery Voltage = 48V

- Energy Discharged = 4,800 Wh

### Step 1: Calculate DoD

DoD%=4800200×48×100 DoD=48009600×100=50%

### Step 2: Calculate SoC

SoC%=100-50=50%

# 5. Standards & References

- IEC 61427 - Performance and endurance testing for batteries in renewable energy applications

- IEEE 485 - Recommended practices for sizing lead-acid batteries

- IEC 62619 - Safety standards for secondary lithium batteries

# 6. Conclusion

This calculation is essential for battery management in small- and large-scale solar power plants. Ensuring proper DoD limits and monitoring SoC levels increases battery efficiency and lifespan.