Snow Load & Structural Load Calculation for Rooftop & Ground-Mount Solar Systems

## 1. Introduction

Snow load and structural load calculations are critical for ensuring the stability and safety of rooftop and ground-mounted solar power plants. This document outlines the calculations and relevant standards (IS 875 Part-4, IEC 61215, ASCE 7-16) to determine the effects of snow load on solar panel structures.

## 2. Snow Load Calculation

### Formula for Snow Load (S)

The snow load on a structure is calculated using: S=Cs×Is×Pg Where:

- S = Snow load on the solar panel (kN/m²)

- C_s = Snow load shape coefficient (from IS 875 Part-4 or ASCE 7-16)

- I_s = Importance factor (from codes, typically 1.0 - 1.2)

- P_g = Ground snow load (kN/m², obtained from local meteorological data)

### Example Calculation

For a solar panel with:

- Ground snow load (P_g) = 2.5 kN/m²

- Shape coefficient (C_s) = 0.8 (for a sloped surface)

- Importance factor (I_s) = 1.1

S=0.8×1.1×2.5=2.2kN/m²

## 3. Structural Load Calculation

### Total Load on Solar Panels

The total structural load includes dead load, wind load, and snow load.

Wt=Wd+Ww+S Where:

- W_t = Total structural load (kN/m²)

- W_d = Dead load (weight of solar panels & mounting structure, kN/m²)

- W_w = Wind load (calculated as per IS 875 Part-3, kN/m²)

- S = Snow load (calculated above)

### Example Calculation

For a rooftop solar installation:

- Dead load (W_d) = 0.3 kN/m²

- Wind load (W_w) = 1.2 kN/m²

- Snow load (S) = 2.2 kN/m²

Wt=0.3+1.2+2.2=3.7kN/m²

## 4. Structural Safety Check

The calculated structural load is compared against the allowable load capacity of the roof or ground mounting structure.

### Load-Bearing Capacity Formula

F=σSf Where:

- F = Allowable stress (kN/m²)

- σ = Yield strength of structural material (steel, aluminum, etc.)

- S_f = Safety factor (typically 1.5 - 2.0 as per IS 800/ASCE 7-16)

If W_t ≤ F, the structure is safe; otherwise, reinforcement is required.

## 5. Compliance with Standards

The calculations must adhere to:

- IS 875 Part-4 – Snow loads for building structures

- IS 875 Part-3 – Wind load considerations

- IEC 61215 – Solar module mechanical loading

- ASCE 7-16 – Structural design standards

## 6. Conclusion

This document provides a systematic approach to determining snow and structural loads for solar power plants. Ensuring compliance with IS and IEC standards will enhance safety and performance.