# Wind Load Calculation on Solar Panels (As per IS 875 Part-3)

## 1. Introduction

Wind load calculation is essential for designing secure and durable solar panel installations, ensuring they can withstand wind forces as per IS 875 (Part-3). This document provides a detailed approach to calculating wind loads on solar panels.

## 2. Wind Load Calculation Formula

The wind force on a solar panel is calculated using the equation:

F = C_d \times A \times Pz

Where:

- F = Wind force (N)

- C_d = Drag coefficient (dimensionless, typically 1.2-2.0 for flat surfaces)

- A = Exposed area of the solar panel (m²)

- Pz = Wind pressure at height z (N/m²)

### Wind Pressure Calculation (Pz)

Wind pressure is determined using:

Pz = 0.6 \times Vz²

Where:

- Pz = Wind pressure at height z (N/m²)

- Vz = Design wind speed at height z (m/s)

### Design Wind Speed (Vz)

The design wind speed is given by:

Vz = Vb \times k1 \times k2 \times k3

Where:

- Vb = Basic wind speed (m/s) as per IS 875 Part-3

- k1 = Probability factor (1.0 for general use)

- k2 = Terrain, height, and structure size factor

- k3 = Topography factor

## 3. Table of Wind Pressure Coefficients

| Wind Speed (m/s) | Wind Pressure (N/m²) |
| --- | --- |
| 33 | 653.4 |
| 39 | 912.6 |
| 44 | 1161.6 |
| 47 | 1323.6 |
| 50 | 1500.0 |

## 4. Example Calculation

Given Data:

- Basic wind speed (Vb): 50 m/s (as per IS 875 Part-3 for coastal areas)

- Panel dimensions: 2m x 1m (Area = 2m²)

- Height (z): 10m

- k1: 1.0

- k2: 1.05

- k3: 1.1

- C_d: 1.5

### Step 1: Calculate Design Wind Speed

Vz = 50 \times 1.0 \times 1.05 \times 1.1 = 57.75 m/s

### Step 2: Calculate Wind Pressure

Pz = 0.6 \times (57.75)^2 = 2001.53 N/m²

### Step 3: Calculate Wind Force

F = 1.5 \times 2 \times 2001.53 = 6004.6 N

## 5. Conclusion

This calculation ensures that solar panel structures are designed to withstand wind loads according to IS 875 Part-3. Proper design considerations, including wind speed, pressure, and structural strength, are critical for long-term stability and safety.